O-RINGS & SEALS  1

Auto parts stores sell only Buna-N (Nitrile), 70 durometer (This is a measure of resiliency, 70 being softer than 90), temperature range is -40 Deg F to +275 Deg F (-40 Deg C to +135 Deg C).

I use only VITON o-rings, 75 durometer (75 AND 90 being commonly available). Temperature range is -5 Deg F to +395 Deg F (-20 Deg C to +200 Deg C).  Viton o-rings are Dowty compound number 9775.

Contact Dowty Corp., Sully Road, PO Box 5000, Sterling, VA 22170, 703-450-8200, 803-747-7656 for catalog and nearest metric distributor.

The following chart lists available o-ring diameters and cross-section areas: (inch x 25.4 = mm;  mm / 25.4 = inch)

INCH	   MM    AREA IN  AREA MM     INCH    MM    AREA IN  AREA MM

─────────────────────────────      ──────────────────────────────

.040	  1,02   .00133   .03378	     .103   2,62   .00833   .21158

.050	  1,27   .00196   .04978     		  2,70   .00882   .22403

1,50   .00273   .06934		       3,00   .01094   .27788

.060	  1,53   .00283   .07188			  3,50   .01496   .37998

1,60   .00312   .07925		.139	  3,53   .01517   .38532

.070	  1,78   .00385   .09779			  3,60   .01561   .39649

1,90   .00442   .11227			  4,00   .01961   .49809

2,00   .00490   .12446			  5,00   .03048   .77419

2,40   .00709   .18009		.210	  5,34   .03464   .87986

2,50   .00754   .19152			  5,70   .03941  1.00101

.275	  6,99   .05940  1.50876

In all applications, o-ring cross-section area must always be smaller than o-ring groove cross-section area.  Groove area = width x depth.

In plate to plate sealing applications, o-ring diameter must be larger than groove depth and as large as possible is best.

In static shaft sealing applications, o-ring diameter should be slightly greater than groove depth. Ignore groove width.

Coat o-rings lightly with clean vaseline when installing.  This will help hold them in place during assembly and will also aid sealing.

Of course, grooves and mating surfaces must be clean.  Fine steel wool is good for polishing but care must be taken with sealing surfaces.  Scratches that cross the o-ring contact path can cause leaks. Always polish in a circular motion about the o-ring center.

O-RINGS & SEALS  2

SUPPLIERS

Allied Metrics, Inc.			Metric seals & o-rings

7 Ridge Road

Oak Ridge, NJ 07438

201-697-2679

201-697-1224 FAX

Kyle Fleming

5399 Sir Barton Drive

Virginia Beach, VA 23462

804-499-5170

Group One Sales

P.O. Box 3309

Fort Lee, N. J. 07024

201-224-4656

Maserati Information Exchange

P.O. Box 772

Mercer Island, WA 98040

206-455-4449

800-627-3728

O-RINGS & SEALS  3

EARLY INTAKE MANIFOLD WATER	GROOVE  (in) 0.705 x 1.100 x ?

GHIBLI: TABLE 1, ITEM 11			   (mm) 17,91 x 27,94 x ?

BOOK SIZE    18,68 x 25,74 x 3,53

QTY		8				BUY		   SAME

PART NO.	GOM 61090			PART NO. 	   200-210-9775

───────────────────────────────────────────────────────────────

EARLY INTAKE MANIFOLD AIR 	GROOVE  (in) 1.475 x 1.840 x ?

GHIBLI: TABLE 1, ITEM 12			   (mm) 37,47 x 46,74 x ?

BOOK SIZE	   37,70 x 44,76 x 3,53

QTY		8				BUY		   SAME

PART NO.	GOM 66429			PART NO.	   200-222-9775

───────────────────────────────────────────────────────────────

LATE INTAKE MANIFOLD WATER	GROOVE  (in) 1.356 x 1.750 x .096

BORA:  TABLE 1, ITEM 11			   (mm) 34,44 x 44,45 x 2,44

BOOK SIZE    34,52 x 41,85 x 3,53

QTY		2				BUY		   SAME

PART NO.	GOM 65805			PART NO. 	   200-220-9775

200-221-9775 (McClure)

───────────────────────────────────────────────────────────────

LATE INTAKE MANIFOLD AIR 	GROOVE  (in) 1.615 x 2.047 x .139

BORA:  TABLE 1, ITEM 12			   (mm) 41,02 x 52,00 x 3,53

BOOK SIZE	   44,45 x 51,51 x 3,53

QTY		8				BUY		   44,05 x 51,11 x 3,53

PART NO.	GOM 66427			PART NO.	   200-224-9775

200-223-9775 (McClure)

───────────────────────────────────────────────────────────────

OIL FILL CAP				GROOVE  (in) --------------------

BORA:  TABLE 1, ITEM 49	 		   (mm) --------------------

BOOK SIZE	   32,00 x 38,00 x 3,00

QTY		1				BUY		   SAME

PART NO.	GOM 42352			PART NO.	   206-332-9775

───────────────────────────────────────────────────────────────

FRONT HOUSING OIL SEAL		GROOVE  (in) 0.429 x 0.628 x .085

GHIBLI: TABLE 2, ITEM 81		 	   (mm) 10,89 x 15,95 x 2,16

BORA    TABLE 2, ITEM 68		BOOK SIZE	   11,00 x 16,00 x 2,50

QTY		2				BUY		   SAME

PART NO.	GOM 42229			PART NO.	   206-111-9775

───────────────────────────────────────────────────────────────

CYLINDER LINER				GROOVE  (in) --------------------

GHIBLI: TABLE 2, ITEM 48		 	   (mm) --------------------

BORA:   TABLE 2, ITEM 39		BOOK SIZE	   70,00 x 67,60 x 1,20

QTY		8				BUY		   70,00 x 67,00 x 1,50

PART NO.	GOM 50315			PART NO.	   206-070-9775

───────────────────────────────────────────────────────────────

OIL FITTING SEAL			GROOVE  (in) 2.943 x 3.179 x .076

GHIBLI: TABLE 2, ITEM 20		 	   (mm) 74,75 x 80,75 x 1,93

BOOK SIZE	   75,00 x 80,00 x 2,50

QTY		1				BUY		   SAME

PART NO.	GOM 52319			PART NO.	   206-175-9775

O-RINGS & SEALS  4

CRANKSHAFT EXTENSION		GROOVE  (in) 0.978 x 1.112 x .065

BORA: TABLE 3, ITEM 30			   (mm) 24,84 x 28,25 x 1,65

BOOK SIZE    NONE

QTY		1				BUY		   24,00 x 28,00 x 2,00

PART NO.	GOM 55510			PART NO.	   204-024-9775

───────────────────────────────────────────────────────────────

CHAIN TENSIONER HOUSING		GROOVE  (in) 1.534 x 1.734 x .130

GHIBLI: TABLE 4, ITEM 24			   (mm) 38,96 x 44,04 x 3,30

BORA:   TABLE 4, ITEM 23		BOOK SIZE	   39,00 x 45,00 x 3,00

QTY		2				BUY		   SAME

PART NO.	GOM 49497			PART NO.	   206-339-9775

───────────────────────────────────────────────────────────────

CHAIN TENSIONER SHAFT		GROOVE  (in) 0.709 x ?     x .115

GHIBLI: TABLE 4, ITEM 25			   (mm) 18,01 x ?     x 2,92

BORA  : TABLE 4, ITEM 24		BOOK SIZE    18,00 x 24,00 x 3,00

QTY		2				BUY		   SAME

PART NO.	GOM 45791			PART NO.	   206-318-9775

───────────────────────────────────────────────────────────────

MAIN CHAIN IDLER SHAFT		GROOVE  (in) 0.641 x 0.786 x .082

GHIBLI: TABLE 4, ITEM 63			   (mm) 16,28 x 19,96 x 2,08

BORA  : TABLE 4, ITEM 60		BOOK SIZE	   14,20 x 18,60 x 2,20

QTY		1				BUY		   15,60 x 19,16 x 1,78

PART NO.	GOM 57307			PART NO.	   200-016-9775

───────────────────────────────────────────────────────────────

OIL FILL CAP				GROOVE  (in) --------------------

GHIBLI: TABLE 5, ITEM 73 		   (mm) --------------------

BOOK SIZE	   NONE

QTY		1				BUY

PART NO.	GOM 50656			PART NO.

───────────────────────────────────────────────────────────────

OIL FILTER HOUSING			GROOVE  (in) 1.100 x 1.335 x .100

GHIBLI: TABLE 5, ITEM 53			   (mm) 27,94 x 33,91 x 2,54

BORA  : TABLE 5, ITEM 21		BOOK SIZE	   28,00 x 34,00 x 3,00

QTY		2				BUY		   SAME

PART NO.	GOM 42693			PART NO.	   206-328-9775

───────────────────────────────────────────────────────────────

OIL FILTER HOUSING			GROOVE  (in) 0.874 x 1.100 x .100

GHIBLI: TABLE 5, ITEM 54			   (mm) 22,20 x 27,94 x 2,54

BORA  : TABLE 5, ITEM 22		BOOK SIZE    22,00 x 28,00 x 3,00

QTY		1				BUY		   SAME

PART NO.	GOM 42866			PART NO.	   206-322-9775

───────────────────────────────────────────────────────────────

OIL PUMP HOUSING			GROOVE  (in) ?

GHIBLI: TABLE 5, ITEM 18			   (mm) ?

BORA  : TABLE 5, ITEM 84		BOOK SIZE    70,00 x 72,40 x 1,20

QTY		1				BUY		   70,00 x 73,00 x 1,50

PART NO.	GOM 50315			PART NO.	   206-080-9775

O-RINGS & SEALS  5

OIL PICK-UP TUBE			GROOVE  (in) 1.361 x 1.579 x .087

GHIBLI: TABLE 5, ITEM 44			   (mm) 34,57 x 40,11 x 2,21

BORA  : TABLE 5, ITEM 88		BOOK SIZE    34,60 x 39,70 x 3,65

QTY		1				BUY		   34,60 x 39,84 x 2,62

PART NO.	GOM 54022			PART NO.	   200-126-9775

───────────────────────────────────────────────────────────────

OIL PUMP INLET & OUTLET		GROOVE  (in) ?

GHIBLI: TABLE 5, ITEM 19			   (mm) ?

BOOK SIZE    30,00 x 36,00 x 3,00

QTY		2				BUY		   SAME

PART NO.	GOM 45899			PART NO.	   206-330-9775

───────────────────────────────────────────────────────────────

DIPSTICK					GROOVE  (in) 0.275 x 0.400 x .090

GHIBLI: TABLE 5, ITEM 76			   (mm) 07,01 x 10,16 x 2,29

BOOK SIZE    NONE

QTY		1				BUY		   0.266 x 0.406 x .070

PART NO.	GOM 63446			PART NO.	   200-610-9775

───────────────────────────────────────────────────────────────

WATER PUMP HOUSING			GROOVE  (in) 3.300 x 3.510 x .103

BORA: TABLE 6, ITEM	52		  	   (mm) 83,82 x 89,15 x 2,62

BOOK SIZE	   82,00 x 89,03 x 3,80

QTY		1				BUY		   83,80 x 89,03 x 2,62

PART NO.	GOM 68560			PART NO.	   200-643-9775

───────────────────────────────────────────────────────────────

WATER PUMP TO BLOCK			GROOVE  (in) 1.494 x 1.735 x .093

GHIBLI: TABLE 6, ITEM 15			   (mm) 37,94 x 44,07 x 2,36

BORA  : TABLE 6, ITEM 67		BOOK SIZE    38,00 x 44,00 x 3,00

QTY		1				BUY		   SAME

PART NO.	GOM 41462			PART NO.	   206-338-9775

───────────────────────────────────────────────────────────────

DISTRIBUTOR SHAFT			GROOVE  (in) ?

GHIBLI: TABLE 8, ITEM 77			   (mm) ?

BORA  : TABLE 7, ITEM 85		BOOK SIZE    21,00 x 27,00 x 3,00

QTY		1				BUY		   SAME

PART NO.	GOM 51396			PART NO.	   206-321-9775

───────────────────────────────────────────────────────────────

THERMOSTAT HOUSING			GROOVE  (in) 3.154 x 3.309 x .036

BORA: TABLE 8, ITEM	41			   (mm) 80,11 x 84,05 x 0,91

BOOK SIZE    NONE

QTY		1				BUY		   80,00 x 83,00 x 1,50

PART NO.	GOM 65802			PART NO.	   206-080-9775

───────────────────────────────────────────────────────────────

THERMOSTAT HOUSING			GROOVE  (in) 2.271 x 2.559 x .072

GHIBLI: TABLE 9, ITEM 55			   (mm) 57,68 x 65,00 x 1,83

BOOK SIZE    58,42 x 63,20 x 2,62

QTY		2				BUY		   SAME

PART NO.	GOM 61411			PART NO.	   200-141-9775 or

206-158-9775

O-RINGS & SEALS  6

TRANSMISSION DIPSTICK		GROOVE  (in) 0.376 x 0.550 x .107

GHIBLI: TABLE 13, ITEM 41		   (mm) 09,55 x 13,97 x 2,72

BOOK SIZE    NONE

QTY		1				BUY		   0.362 x 0.568 x .103

PART NO.	GOM 57918			PART NO.	   200-110-9775

───────────────────────────────────────────────────────────────

OUTER A-ARM BUSHING			GROOVE  (in) --------------------

BORA: TABLE 18, ITEM 43			   (mm) --------------------

BOOK SIZE    28,25 x 33,50 x 2,62

QTY		8				BUY		   SAME

PART NO.	GOM 75774			PART NO.	   200-122-9775

───────────────────────────────────────────────────────────────

INNER A-ARM BUSHING			GROOVE  (in) --------------------

BORA: TABLE 18, ITEM 38			   (mm) --------------------

BOOK SIZE    32,95 x 40,00 x 3,53

QTY		8				BUY		   SAME

PART NO.	GOM 67775			PART NO.	   200-219-9775

───────────────────────────────────────────────────────────────

POWER STEERING PUMP CASE		GROOVE  (in) 2.675 x 2.800 x .130

GHIBLI: TABLE 22, ITEM 78		   (mm) 67,95 x 71,12 x 3,30

BOOK SIZE    NONE

QTY		1				BUY          2.818 x 2.612 x .103

PART NO.	GOM 59894			PART NO.     200-146-9775

───────────────────────────────────────────────────────────────

GAS CAP					GROOVE  (in)

BORA: TABLE 26, ITEM 8			   (mm)

BOOK SIZE    NONE

QTY		1				BUY

PART NO.	GOM 50650			PART NO.

───────────────────────────────────────────────────────────────

GAS CAP					GROOVE  (in)

GHIBLI: TABLE 29, ITEM 39		   (mm)

BOOK SIZE    NONE

QTY		2				BUY

PART NO.	GOM 50656			PART NO.

───────────────────────────────────────────────────────────────

CAM COVER STUD SEAL			GROOVE       Replace rubber inside

GHIBLI: TABLE  , ITEM			        washers

BORA  : TABLE 1, ITEM 25		BOOK SIZE    NONE

QTY		32				BUY		   4,00 x 7,00 x 1,50

PART NO.	CN 65244			PART NO.	   206-004-9775

O-RINGS & SEALS  7

SEAL - ALUMINUM CRUSH WASHERS

Earl's Performance Products		3/8" ID	177003

14611 Hawthorne Blvd.			7/16"	177004

Lawndale, CA 90260				9/16"	177006

310-644-0881, 2298				3/4"		177008

310-644-6475 FAX				7/8"		177010

1-1/16"	177012

1-5/16"	177016

10mm		177100

SEAL - ALUMINUM WASHER WITH O-RING SEAL

Earl's Performance Products		3/16" ID	178003

14611 Hawthorne Blvd.			1/4"		178004

Lawndale, CA 90260				5/16"	178005

310-644-0881, 2298				3/8"		178006

310-644-6475 FAX				7/16"	178007

1/2"		178008

9/16"	178009

5/8"		178010

3/4"		178012

7/8"		178014

NOTE:  Also see ENGINE for additional seals.