ENGINE  1

SUPPLIERS

A & P Bearing

7225 Canoga Avenue, Unit B

Canoga Park, CA 91303

818-716-4003

Allied Metrics, Inc.			Metric seals & o-rings

7 Ridge Road

Oak Ridge, NJ 07438

201-697-2679

201-697-1224 FAX

Antique Auto Parts Cellar		Motor mount re-vulcanization

P.O. Box 3

South Weymouth, MA 02190

617-335-1597

Arias Forged Pistons

13420 S. Normandie Avenue

Gardena, CA 90249-2212

310-532-9737

B.T. Street Rod City			Stainless steel braided hose

16 E. Columbia Avenue

Palisades Park, NJ 07650

201-943-8264

Citroen Concours of America		Engine, brake, hydraulic parts

8180 Miramar Road

San Diego, CA 92126

619-566-2860 Parts

619-566-2861 Tech

CR Services (Chicago Rawhide)		Speedi-SleeveTM shaft repair

735 Tollgate Road				sleeve. Available at Pep Boys;

Elgin, IL 60123				Winston Tire; B&D Brake Parts,

800-882-0008					Bakersfield

708-742-7840

Del West Engineering			Titanium valves, intake $74,

24711 Avenue Rockafeller			exhaust $84 (1991), min order

Valencia, CA 91355				8 valves, Paul Jette

805-295-5700

ENGINE  2

SUPPLIERS (CONTINUED)

Deves Piston Rings				Maria; Maserati V8 rings

137-139 Nevada Street

El Segundo, CA 90245

213-322-8484

E & E Performance				Stainless steel valves, sodium

?, LA						cooled for Maserati (Hemmings

504-486-6383					10/92)

Egge Machine Company			Fabricate pistons and stainless

8403 Allport Avenue #C			steel valves

Santa Fe Springs, CA 90670

310-945-3419

800-866-EGGE

Engine Machine Works			Pistons built up oversize,

N 604 Freya					piston rings, piston cam

Spokane, WA 99202				ground

800-233-6934

Group One Sales				Maserati parts

P.O. Box 3309

Fort Lee, N. J. 07024

201-224-4656

High Mountain Classics			Jim Stranberg, Maserati & Lambo

?							Gaskets, formerly fabricated by

?, CO						Bill Stewart

303-444-9085

Jahns Quality Pistons, Inc.		Forged & cast pistons

1465 W. El Segundo Blvd

Compton, CA 90222

800-231-8661 CA

800-225-0277 USA

JE Pistons, Inc				Barry Calvert, Talk to "Snake"

15312 Connector Lane			TRW forged blanks, high

Huntington Beach, CA 92649-1120	silicon content, lower

714-837-7200					coefficient of expansion,

714-898-9763					moly rings

Kano Laboratories				Kreen Motor Tonic, dissolves

1000 S. Thompson Lane			sludge, varnish and binders

Nashville, TN 37211				in carbon formations

615-833-4101

ENGINE  3

SUPPLIERS

L A Sleeve					Maserati V8 cylinder sleeves

12051 Rivera Road

Santa Fe Springs, CA 90670-2211

310-945-7578

Metal Technology				Metal analysis $75, 1992

19801 Nordhoff					Stan Block

Northridge, CA

818-882-6414

Metro Moulded Parts, Inc.		Cam access rubber half circle

11610 Jay Street

P.O. Box 33130

Minneapolis, MN 55433

800-878-2237 Orders

612-757-0310 Questions

Milano Imports					Maserati/Lamborghini engine

12385 Watsonville Road			parts, rebuilding

Gilroy, CA

408-847-8732

Olson's Gaskets

3059 Opdal Road

Port Orchard, WA 98366

Paeco Industries				Deves piston rings

213 S. 21st Street

Birmingham, AL 35233

205-323-8376

Technovance Corporation			Valve guides, 4.2, 4.7, 4.9

956 West Hyde Park Blvd.			aluminum-silicon-bronze

Inglewood, CA 90302				Intake  #7-5508

213-674-5130					Exhaust #7-5511

Use VW Rabbit valve seals

Thread Kits Company				Spark plug thread repair kit

2530 W. 237th Street			for short, normal & long reach

Torrance, CA 90505				tapered seat and gasket type.

800-372-4477					$29 8/93 Popular Mechanics

Wen Industries					Distributor for Technovance

11310 Hartland Street			Intake valve guide  #VG-5508

North Hollywood, CA 91605		Exhaust valve guide #VG-5511

818-761-6137

ENGINE  4

SUPPLIERS

Wurth						DP-300 Gasket sealant

800-526-5228 USA				Ron Hoyme

800-346-4198 x 109 CA

MACHINING

Apple Hydraulics				Worn shafts restored by

1610 Middle Road				thermal spray weld process:

Calverton, NY 11933				mild steel, bronze, stainless

800-882-7753					steel

516-744-9627

Camshaft Corp of America			Regrind and restore all types

435 2B Air Park Road			of camshafts - fast service

Edgewater, FL 32132

904-426-0670

Castillo's Crankshaft Specialist	Joe & Rudy (Sternburg

14654-1/2 E. Firestone Blvd.		recommendation)

La Mirada, CA 90638

714-523-0321

Crankshaft Works

9510 Owensmouth Avenue

Chatsworth, CA

818-709-4644

Custom Auto Machining &			Al Cortes

Fabrication

14312 Mansel Avenue

Lawndale, CA 90260

213-970-0127

Delta Camshaft					Jerry, camshaft & follower

1938 Tacoma Avenue South			regrinding, rehardening

Tacoma, WA 98402				(Groth recommendation)

206-383-4152

Electronic Chrome & Grinding Co.	Mike Reed

9132 Dice Road

Santa Fe Springs, CA 90670-2545

310-946-6671

ENGINE  5

MACHINING (CONTINUED)

Hydrohead						Line boring

? Hawthorne Blvd.				(Recommended by Slover's)

Torrance, CA

310-371-7587

310-672-6578

Kelly Blockwelders				Blocks & heads welded

12910 West Washington Blvd.		JR 9/95

W. Los Angeles, CA

310-827-6795

E. J. Reynolds					Repair cracked cylinder heads,

2632 E. 13th Place				blocks and housings.

Tulsa, OK 74104				Brochure on request

918-744-0417, 1787 home

Slover's Porting Service			Recommended by Joe, Hawaii

7452 Varna Avenue				Racing

N. Hollywood, CA

818-764-2260, 2367

Sperry Valve Works				Mike Sperry, Dave Vager

2829 Gundry Avenue

Signal Hill, CA 90806

213-988-5960

Vintage Engine Machine Works		Camshaft regrinding

North 604 Freya				Jack H. Bunton

Spokane, WA 99202

509-534-0515

Vintage Racecar Restorations		Kevin Bollonger, Rick Cresse

10953 Tuxford, No. 13			Complete fabrication & machine

Sun Valley, CA 91352			shop facilities

818-768-9723

West Valley Engine Balancing

7841 Alabama #3

Canoga Park, CA 91304

818-999-4355

ENGINE  6

CLEANING

DuPont Metal Prep				Warm water and baking soda to neutralize

Advanced Coating Technology		Hydrofluoric acid

PO Box 1413

Simi Valley, CA 93065

HEAD GASKET

High Mountain Classics			Maserati/Lamborghini head

6655 Arapaho Avenue				gaskets, cam cover gaskets

Boulder, CO 80303				Jim Stranberg

303-444-9085

Molina Gaskets					Custom head gaskets, exhaust

23126 Mariposa Avenue			gaskets, etc., copper &

Torrance, CA 90502				asbestos

310-539-1883

Olson's Gaskets

3059 Opdal

Port Orchard, WA 98366

206-871-1207, 4-9PM

ENGINES

International Auto & Marine

?

?

203-377-3020

Herb Kaplan & son Brad			Has made numerous V8 engine

916-961-3118					modifications

916-489-4802

916-486-4998

ENGINE  7

PARTS

Cam chain, twin link			Mercedes, (shorten as req'd)

Cam access rubber insert 		Metro Moulded Parts #RP 8-G

Rod Bearings					Jaguar XKE 62-65 (Fed. Mogul      #2295) (MIE 4/78)

Cam cover cap nut				BMW 11-14-0-002-193 (Loelinger)

Cam cover washer				Wurth #469-6 (McClure)

Cam cover nut & washer			F.A.F. Motorcars

Cam Cover stud o-ring			1/16 Dia x 5/32 ID x 9/32 OD      NAPA 3-7702

Crankshaft pilot bearing			FAG 6303 (MIE Issue 38)

SKF 6303 RS (Bishop)

Fafnir 303PP C2

Valve stem seal				Peugeot 504, 1975-76 (MIE         Issue 43)

Crank sprocket removal tool		Snap-on #CJ-119A, does not        need heat or impact. (MOCNA)

Motor mount					Vibrachoc 104.432-1A

Front engine bearing, Bora 3/28	Italy NRV ELL40 (McClure)

Crank seal, front, 4.7			CR 50-68-8 USA, 19608

Crank seal, front, 4.9			National 1884 (Young)

Crank seal, rear				GACO MIM5068, ROLT 100 120 12

Piston rings					Deves #1999DO STANDARD for        93.9mm, 4-ring set

Idler sprocket bearing (2)		FAG 6004 C3, 1K 098640

Idler pulley bearing			Fafnir 204PP, SKF 6204-2RSJEM

Intake valve guide				Technovance #7-5508

Exhaust valve guide				Technovance #7-5511

Valve stem seal for above guides	AUDI 2.1 Liter Gas 1978-82,

VW Rabbit

ENGINE  8

ASSEMBLY PRODUCTS

Crankcase studs, pump shaft 		Loctite 271 high strength

retaining plate, camshaft

holding nuts, prevents bearing

spinout

Crankshaft pulley 				Loctite 601

Con-rod nuts, crankshaft plugs	Loctite 242 medium strength

Application after assembly of 	Loctite 290 medium strength

threaded parts, penetrates

Bearing retention				Loctite RC/609

Head gasket seal rings			K&W Copper Coat Gasket

Compound

All other gaskets including		Wurth DP-300 (Hylomar)

head gasket					  Remains soft

Yamaha (motorcycle) Yamabond-4

Large clearances, heat,         pressure, hardening

Loctite 819 non hardening, easy    gasket removal (Hylomar)

Replace gaskets with:			Locktite 518 Gasket Eliminator

Cleaner						Brake parts cleaner

Wax & grease remover

Bearing clearance gauges			Plastigage PG-1 Green

.001 to .003

Plastigage PR-1 Red

.002 to .006

Crankshaft, camshaft bearings/	Sta-Lube Moly-Graph

lobes, gears, valve stems		  Engine Assembly Lube

General assembly lube			GM 1#1052367 Engine Oil

Supplement Concentrate

High temp lube/anti-seize		VHT 2000 Plus

Fasten loose studs				Loctite 242 Medium Strength

Repair stripped threaded holes	Loctite Form-A-Thread 81668

ENGINE  9

ARTICLES

MIE		Nov-Dec 1978,  Pg 23	Valve reconditioning

MIE		Mar-Apr 1979,  Pg 25	Connecting rod specifications

MIE		May-June 1979, Pg 32	Timing & valve setting

MIE		Sept-Oct 1979, Pg 26	Pistons

MIE		Sept-Oct 1980, Pg 36	Engine removal & rebuild

MIE		Sept-Oct 1980, Pg 43	Specifications & bolt torques

MIE		Jan-Feb 1981,  Pg 36	Valve adjustment procedure

MIE		May-June 1981, Pg 7		Valve adjustment procedure

MIE		May-June 1981, Pg 37	Bora flat rate repair times

MIE		Sept-Oct 1981, Pg 41	Specifications & bolt torques

MIE		Sept-Oct 1981, Pg 39	Bora engine & subframe removal

MIE		Nov-Dec 1981,  Pg 40	Valve adjustment procedure

MOCNA	Nov 1983,    Pg 14		Bora engine removal

MIE		Issue 42,    Pg 7		Sleeve removal

MIE		Issue 44,    Pg 42		V8 engine notes

MIE		Issue 45,    Pg 54		Fitting & adj timing chains

MIE		Issue 45,    Pg 53		V8 cam timing

MIE		Issue 46,    Pg 57		Bora valve clearance

MIE		Issue 47,    Pg 71		Engine I.D. numbers

MIE		Issue 47,    Pg 68		Cylinder liners

MIE		Issue 47,    Pg 70		Head torquing

MIE		Issue 48,    Pg 75		V8 camshaft specifications

MIE		Issue 49,    Pg 67		V8 engine specifications

MOCNA	Summer 1986, Pg 16		Bora engine overhaul (Part 1)

MOCNA	Fall 1986,   Pg 17		Bora engine overhaul (Part 2)

MOCNA	Spring 1987, Pg 18		V8 cam timing changes

MOCNA	Winter 1987, Pg 13		Cylinder leakdown test

MOCNA	Winter 1987, Pg 14		Cylinder head removal (6 CYL)

MIE		Issue 52,    Pg 69		Engine assembly oil

MIE		Issue 54,    Pg 75		Timing chain tension

MOCNA	Winter 1987, Pg 12		Seal oil leaks

MOCNA	Winter 1988, Pg 15		Bora engine overhaul (Part 3)

MOCNA	Winter 1989, Pg 13		Valve clearance adjustment

MOCNA	Spring 1990, Pg 25		Rebuilding 6 cylinder head

MOCNA	Fall 1990,   Pg 14		Bora Ramblings - cam timing

MOCNA	Fall 1991,   Pg 13		V8 valve seals

MOCNA	Summer 1992, Pg 18		Early Ghibli engine assy 1

MOCNA	Fall 1992,   Pg 16		Early Ghibli engine assy 2

MOCNA	Winter 1993, Pg 16		Early Ghibli engine assy 3

MOCNA	Fall 1992,   Pg 22		V8 bearing scarcity

MIE		Issue 62,    Pg 82		V8 valve guide seal size

CAR*		August 1993, Pg 		Aluminum casting restoration

CAR*		October 1993,Pg 		Broken stud/bolt repair

MIE		Issue 65,    Pg 72		Cam cover sealing

MOCNA	Summer 1994, Pg 20		Maintenance of fluids

MOCNA	Spring 1996, Pg 13		Eliminating cam cover oil leaks

(* Classic Auto Restorer)

ENGINE  10

ARTICLES

MOCNA	Summer 1996, Pg 13		Camshaft repair

ENGINE - SPECIFICATIONS  1

NOTE:  Much of the data is from the Quattroporte III manual

BLOCK

Compression at operating			Good, over	11-12 bar

temperature, throttle			Normal		9-10 bar

open, starter speed				Bad,lower than 8 bar

Firing order					1-8-4-2-7-3-6-5

Crankshaft rotation:  clockwise as seen from the front

Cylinder liner interference fit  				.0008

Fitting is done by heating the cylinder block

to 160 degrees C (320F).

Cylinder liner surface roughness, microinch		30-40

Cylinder liner maximum ovalization with

tightening plate						.0012

Cylinder liner wear limit from basic value		.0012-.0016

Piston/liner clearance, at skirt				.0020-.0024

Piston groove/ring clearance, all				.0008-.0020

Piston ring gap, upper and center				.0177-.0217

Piston ring gap, oil scraper					.0138-.0157

Piston pin/bushing clearance					.0004-.0006

Crankshaft main bearing clearance				.0020

Crankshaft/connecting rod bearing clearance		.0024

Crankshaft end float						.0079 maximum

Crankshaft eccentricity						.0006 maximum

ENGINE - SPECIFICATIONS  2

BLOCK (CONTINUED)

Check crankshaft/con-rod/piston assembly with the following chart:

4700 cc						  4900 cc

CRANKSHAFT      PISTON			   CRANKSHAFT      PISTON

ROTATION	     LOWERING			    ROTATION	   LOWERING

deg.	   mm	  in.			 deg.	 mm		in.

2 	0.012	.0004	2	0.055	.0022

3 	0.066	.0026	3	0.090	.0035

4 	0.132	.0052	4	0.120	.0047

5 	0.215	.0085	5	0.240	.0094

6 	0.297	.0117	6	0.330	.0130

7 	0.412	.0162	7	0.450	.0177

8 	0.528	.0208	8	0.550	.0217

9 	0.660	.0260	9	0.700	.0276

10	0.820	.0323	10	0.880	.0346

11	1.000	.0394	11	1.060	.0417

12	1.181	.0465	12	1.300	.0512

13	1.386	.0546	13	1.520	.0598

14	1.590	.0626	14	1.740	.0685

15	1.841	.0725	15	1.950	.0768

16	2.099	.0826	16	2.250	.0886

17	2.349	.0925	17	2.450	.0965

18	2.640	.1039	18	2.780	.1094

19	2.937	.1156	19	3.080	.1213

20	3.240	.1276	20	3.400	.1339

21	3.560	.1402	21	3.750	.1476

22	3.894	.1533	22	4.100	.1614

23	4.257	.1676	23	4.530	.1783

24	4.666	.1837	24	4.850	.1909

25	5.050	.1988	25	5.150	.2028

ENGINE - SPECIFICATIONS  3

HEAD

Head surface flatness difference				.0012 maximum

Valve seat interference fit, intake			.0059-.0063

Valve seat interference fit, exhaust			.0055-.0059

(Fitting is done by heating the head to 180

degrees C and cooling the seat to -70 C)

Valve guide interference fit 					.0008-.0012

Valve stem/guide clearance, intake				.0012-.0020 .0024 maximum

Valve stem/guide clearance, exhaust			.0020-.0024 .0028 maximum

Valve stem eccentricity						.0012 maximum

Valve surface on seat						.0787 minimum

Valve seat angle (4700 cc)					35 degrees

(4900 cc)					45 degrees

Top & throat angle difference (3 angle job)		15 degrees min.

Free minimum valve spring height (approx)		1.811

Installed valve spring height (approx)			1.063

Inner valve spring compression of 1.240			29 Kg ± 15%

Outer valve spring compression of 1.378			48 Kg ± 15%

Camshaft end clearance						.0039-.0059

Camshaft bearing clearance					.0020-.0024

ENGINE - SPECIFICATIONS  4

HEAD (CONTINUED)

CAMSHAFT 67000, INTAKE:

Dimensions: Diameter					1.024 (26mm)1

Lift						0.433 (11mm)1

Cam lobe to opposite side		1.457 (37mm)1

Markings: Right (destra = D)				TD ASP

Long line						T.D.C. #1

Left  (sinistra = S)			TS ASP

Long line						T.D.C. #1

Short line					T.D.C. #8

Engine 107/47/3 - 4700cc (Small batch)2

Engine 115/0340 - 4900cc2

Valve opens						40 deg B.T.D.C.

Valve closes						80 deg A.B.D.C.

Duration of valve open				300 degrees

Valve clearance					.011 (0.28mm)

Valve opening at T.D.C.		 		.087 (2.2mm)

Engine 115/0549 - 4900cc (Air polution 1971)2

Valve opens						38 deg B.T.D.C.

Valve closes						84 deg A.B.D.C.

Duration of valve open				302 degrees

Valve clearance					.011 (0.28mm)

Valve opening at T.D.C.				.071 (1.8mm)

Engine 107/07/47 - 4700cc Bora2

Engine ?         - 4200cc4

Valve opens						40 deg B.T.D.C.

Valve closes						80 deg A.B.D.C.

Duration of valve open				300 degrees

Valve clearance					.011 (0.28mm)

Valve opening at T.D.C.				.075 (1.9mm)

Engine ?         - 4900cc Bora5

Valve opens						52 deg B.T.D.C.

Valve closes						68 deg A.B.D.C.

Duration of valve open				300 degrees

Valve clearance					.011 (0.28mm)

Valve opening at T.D.C.				.075 (1.9mm)

ENGINE - SPECIFICATIONS  5

HEAD (CONTINUED)

CAMSHAFT 67000, EXHAUST:

Dimensions: Diameter					1.024 (26mm)1

Lift						0.433 (11mm)1

Cam lobe to opposite side		1.457 (37mm)1

Markings: Right						TD SCA

Long line						T.D.C. #1

Left							TS SCA

Long line						T.D.C. #1

Short line					T.D.C. #8

Engine 107/07/47 - 4700cc Bora (Small batch)2

Valve opens						?? deg B.B.D.C.

Valve closes						?? deg A.T.D.C.

Duration of valve open				??? degrees

Valve clearance					.???

Valve opening at T.D.C.				.067 (1.7mm)

ENGINE - SPECIFICATIONS  6

HEAD (CONTINUED)

CAMSHAFT 67500, EXHAUST:

Dimensions: Diameter					1.024   (26mm)1

Lift						0.402 (10.2mm)1

Cam lobe to opposite side		1.425 (36.2mm)1

Markings: Right						TD SCA

Long line						T.D.C. #1

Left							TS SCA

Long line						T.D.C. #1

Short line					T.D.C. #8

Engine 107/47/3  - 4700cc (Small batch)2

Engine 115/0340  - 4900cc2

Engine 107/07/47 - 4700cc Bora2

Engine ?         - 4200cc4

Valve opens						54 deg B.B.D.C.

Valve closes						22 deg A.T.D.C.

Duration of valve open				256 degrees

Valve clearance					.019 (0.47mm)

Valve opening at T.D.C.				.067 (1.7mm)

Engine 115/0549  - 4900cc (Air polution 1971)2

Valve opens						65 deg B.B.D.C.

Valve closes						10 deg A.T.D.C.

Duration of valve open				255 degrees

Valve clearance					.019 (0.47mm)

Valve opening at T.D.C.				.016 (0.4mm)

Engine ?         - 4900cc Bora5

Valve opens						63 deg B.B.D.C.

Valve closes						13 deg A.T.D.C.

Duration of valve open				256 degrees

Valve clearance					.019 (0.47mm)

Valve opening at T.D.C.				.067 (1.7mm)

ENGINE - SPECIFICATIONS  7

HEAD (CONTINUED)

CAMSHAFT 48377, INTAKE:

Dimensions: Diameter					1.024   (26mm)1

Lift						0.375 (9.52mm)1

Cam lobe to opposite side		1.398 (35.5mm)1

Markings: Right (destra = D)				TD ASP

Long line						T.D.C. #1

Left  (sinistra = S)			TS ASP

Long line						T.D.C. #1

Short line					T.D.C. #8

Engine 107/1, 107/2, 107/3 - 4200cc2

Engine 107/47/1, 107/47/2, 107/47/3 - 4700cc2

Valve opens						34 deg B.T.D.C.

Valve closes						66 deg A.B.D.C.

Duration of valve open				280 degrees

Valve clearance		(0.15-0.17mm)	.006-.007

Valve opening at T.D.C.				.051 (1.3mm)

Engine 115 - 4700cc (Air polution 1970)2

Valve opens						15 deg B.T.D.C.

Valve closes						57 deg A.B.D.C.

Duration of valve open				252 degrees

Valve clearance					.011 (0.28mm)

Valve opening at T.D.C.				.0315 (0.8mm)

Engine ?         - 4700cc3

Valve opens						36 deg B.T.D.C.

Valve closes						66 deg A.B.D.C.

Duration of valve open				282 degrees

Valve clearance					.011 (0.28mm)

Valve opening at T.D.C.				.051 (1.3mm)

ENGINE - SPECIFICATIONS  8

HEAD (CONTINUED)

CAMSHAFT 48377, EXHAUST:

Dimensions: Diameter					1.024   (26mm)1

Lift						0.375 (9.52mm)1

Cam lobe to opposite side		1.398 (35.5mm)1

Markings: Right (destra = D)				TD SCA

Long line						T.D.C. #1

Left  (sinistra = S)			TS SCA

Long line						T.D.C. #1

Short line					T.D.C. #8

Engine 107/1 - 4200cc2

Engine 107/47/1 - 4700cc2

Valve opens						88 deg B.B.D.C.

Valve closes						45 deg A.T.D.C.

Duration of valve open				313 degrees

Valve clearance		(0.40-0.42mm)	.0157-.0165

Valve opening at T.D.C.				.067 (1.7mm)

Engine ?        - ????cc1

Valve opens						?? deg B.B.D.C.

Valve closes						?? deg A.T.D.C.

Duration of valve open				??? degrees

Valve clearance		(0.30-0.35mm)	.0118-.0138

Valve opening at T.D.C.				.043 (1.1mm)

ENGINE - SPECIFICATIONS  9

HEAD (CONTINUED)

CAMSHAFT 64400, EXHAUST:

Dimensions: Diameter					1.024   (26mm)1

Lift						0.359 (9.11mm)1

Cam lobe to opposite side		1.382(35.11mm)1

Markings: Right						TD SCA

Long line						T.D.C. #1

Left							TS SCA

Long line						T.D.C. #1

Short line					T.D.C. #8

Engine 107/2, 107/3 - 4200cc2

Engine 107/47/2, 107/47/3 - 4700cc2

Valve opens						70 deg B.B.D.C.

Valve closes						30 deg A.T.D.C.

Duration of valve open				280 degrees

Valve clearance		(0.45-0.47mm)	.018-.019

Valve opening at T.D.C.				.067 (1.7mm)

Engine 115 - 4700cc (Air polution 1970)2

Valve opens						88 deg B.B.D.C.

Valve closes						19 deg A.T.D.C.

Duration of valve open				287 degrees

Valve clearance					.0185 (0.47mm)

Valve opening at T.D.C.		 		.024 (0.6mm)

Engine ?         - 4700cc3

Valve opens						88 deg B.B.D.C.

Valve closes						45 deg A.T.D.C.

Duration of valve open				313 degrees

Valve clearance					.020 (0.50mm)

Valve opening at T.D.C.		 		.067 (1.7mm)

Camshaft data sources:

1	MIE, Issue 48, Page 75

2	MIE, Issue 49, Page 67

3	MIE, Vol 4 #5, Page 43

4	MIE, Vol 5 #5, Page 41

5	MIE, Nov/Dec 81, Pg 40

CHAINS

Main timing chain: 		Triplex roller			.250 dia

Pitch				.375

Number of links		60 (22.44 lg)

Cam timing chain right:	Duplex roller			.250 dia

Pitch				.375 mm

Number of links		134 (50.27)

Cam timing chain left:	Duplex roller			.250 dia

Pitch				.375

Number of links		116 (43.50)

(Also see TORQUE SPECIFICATIONS)

ENGINE - VALVE LASH CLEARANCE WORKSHEET

DATE:							  1   2   3   4

┌─────────────┐

CAR:   						FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

Intake Camshaft  #67000: .28 - .30mm (.0110 - .0118 in.)

Exhaust Camshaft #67500: .45 - .48mm (.0177 - .0189 in.)

┌───────┬──────────────────────────────────┬──────┬──────┬──────┐

│ RIGHT │ EXIST        TOTAL  FINAL   NEW  │CHECK │ NEW  │ RE   │

│ HEAD  │ SHIM + CLR  = CLR  - CLR  = SHIM │ CLR  │ SHIM │CHECK │

├───────┼──────┬──────┬──────┬──────┬──────┼──────┼──────┼──────┤ │ INT 1 │      │      │      │ .012 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ INT 2 │      │      │      │ .012 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ INT 3 │      │      │      │ .012 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ INT 4 │      │      │      │ .012 │      │      │      │      │

├───────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ EXH 1 │      │      │      │ .020 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ EXH 2 │      │      │      │ .020 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ EXH 3 │      │      │      │ .020 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ EXH 4 │      │      │      │ .020 │      │      │      │      │

└───────┴──────┴──────┴──────┴──────┴──────┴──────┴──────┴──────┘

┌───────┬──────────────────────────────────┬──────┬──────┬──────┐

│ LEFT  │ EXIST        TOTAL  FINAL   NEW  │CHECK │ NEW  │ RE   │

│ HEAD  │ SHIM + CLR  = CLR  - CLR  = SHIM │ CLR  │ SHIM │CHECK │

├───────┼──────┬──────┬──────┬──────┬──────┼──────┼──────┼──────┤

│ INT 5 │      │      │      │ .012 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ INT 6 │      │      │      │ .012 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ INT 7 │      │      │      │ .012 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ INT 8 │      │      │      │ .012 │      │      │      │      │

├───────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ EXH 5 │      │      │      │ .020 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ EXH 6 │      │      │      │ .020 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ EXH 7 │      │      │      │ .020 │      │      │      │      │

│       ├──────┼──────┼──────┼──────┼──────┼──────┼──────┼──────┤

│ EXH 8 │      │      │      │ .020 │      │      │      │      │

└───────┴──────┴──────┴──────┴──────┴──────┴──────┴──────┴──────┘

ENGINE - CHAIN TENSIONER ADJUSTMENT WORKSHEET

DATE:							      RIGHT

┌─────────────┐

CAR:   						FRONT │    ENGINE   │

└─────────────┘

ENGINE:							       LEFT

NOTES:

1.	Tighten chain to maximum tension, find matching hole and partially insert pin.

2.	Mark hole location in most accurate view with an "M" for Max.

3.	Move pin to second hole back and rotate to insert pin.

4.	Darken circle in same view to show pin location.

5.	Install retainer washer, lock washer and nut.

VIEW TOWARD FRONT OF ENGINE

RIGHT HEAD		       LEFT HEAD

ENGINE - VALVE STEM/GUIDE CLEARANCE WORKSHEET

DATE:							  1   2   3   4

┌─────────────┐

CAR:   						FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Valve stem/guide clearance, intake should be .0012-.0020, .0024 maximum.

2.	Valve stem/guide clearance, exhaust should be .0020-.0024, .0028 maximum.

┌────────┬────────────────────┐   ┌────────┬────────────────────┐ │  LEFT  │                    │   │ RIGHT  │                    │

│  HEAD  │ BORE - STEM = CLR  │   │  HEAD  │ BORE - STEM = CLR  │

├────────┼──────┬──────┬──────┤   ├────────┼──────┬──────┬──────┤

│ INT 1  │      │      │      │   │ INT 1  │      │      │      │

│        ├──────┼──────┼──────┤   │        ├──────┼──────┼──────┤

│ INT 2  │      │      │      │   │ INT 2  │      │      │      │

│        ├──────┼──────┼──────┤   │        ├──────┼──────┼──────┤

│ INT 3  │      │      │      │   │ INT 3  │      │      │      │

│        ├──────┼──────┼──────┤   │        ├──────┼──────┼──────┤

│ INT 4  │      │      │      │   │ INT 4  │      │      │      │

├────────┼──────┼──────┼──────┤   ├────────┼──────┼──────┼──────┤

│ EXH 1  │      │      │      │   │ EXH 1  │      │      │      │

│        ├──────┼──────┼──────┤   │        ├──────┼──────┼──────┤

│ EXH 2  │      │      │      │   │ EXH 2  │      │      │      │

│        ├──────┼──────┼──────┤   │        ├──────┼──────┼──────┤

│ EXH 3  │      │      │      │   │ EXH 3  │      │      │      │

│        ├──────┼──────┼──────┤   │        ├──────┼──────┼──────┤

│ EXH 4  │      │      │      │   │ EXH 4  │      │      │      │

└────────┴──────┴──────┴──────┘   └────────┴──────┴──────┴──────┘

ENGINE - CRANKSHAFT BEARING CLEARANCE RECORD

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Main bearing clearance must be .0020 maximum.

2.	Con-rod bearing clearance must be .0024 maximum.

3.	End float (thrust) clearance must be .0079 maximum.

┌─────────┬─────────────────┐       ┌─────────┬─────────────────┐

│  MAIN   │                 │       │ CON-ROD │                 │

│ BEARING │    CLEARANCE    │       │ BEARING │    CLEARANCE    │

├─────────┼─────────────────┤       ├─────────┼─────────────────┤

│  FRONT  │                 │       │    1    │                 │

│         ├─────────────────┤       │         ├─────────────────┤

│    2    │                 │       │    2    │                 │

│         ├─────────────────┤       │         ├─────────────────┤

│    3    │                 │       │    3    │                 │

│         ├─────────────────┤       │         ├─────────────────┤

│    4    │                 │       │    4    │                 │

│         ├─────────────────┤       │         ├─────────────────┤

│  REAR   │                 │       │    5    │                 │

└─────────┴─────────────────┘       │         ├─────────────────┤

│    6    │                 │

┌─────────┬─────────────────┐       │         ├─────────────────┤

│END FLOAT│    CLEARANCE    │       │    7    │                 │

├─────────┼─────────────────┤       │         ├─────────────────┤

│         │                 │       │    8    │                 │

└─────────┴─────────────────┘       └─────────┴─────────────────┘

ENGINE - CAMSHAFT BEARING CLEARANCE RECORD

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Bearing clearance must be .0020 to .0024.

2.	End float (thrust) clearance must be .0039 to .0059.

┌─────────┬─────────────────┐       ┌─────────┬─────────────────┐

│  LEFT   │                 │       │  RIGHT  │                 │

│ INTAKE  │    CLEARANCE    │       │ INTAKE  │    CLEARANCE    │

├─────────┼─────────────────┤       ├─────────┼─────────────────┤

│  FRONT  │                 │       │  FRONT  │                 │

│         ├─────────────────┤       │         ├─────────────────┤

│ CENTER  │                 │       │ CENTER  │                 │

│         ├─────────────────┤       │         ├─────────────────┤

│  REAR   │                 │       │  REAR   │                 │

├─────────┼─────────────────┤       ├─────────┼─────────────────┤

│ THRUST  │                 │       │ THRUST  │                 │

└─────────┴─────────────────┘       └─────────┴─────────────────┘

┌─────────┬─────────────────┐       ┌─────────┬─────────────────┐

│  LEFT   │                 │       │  RIGHT  │                 │

│ EXHAUST │    CLEARANCE    │       │ EXHAUST │    CLEARANCE    │

├─────────┼─────────────────┤       ├─────────┼─────────────────┤

│  FRONT  │                 │       │  FRONT  │                 │

│         ├─────────────────┤       │         ├─────────────────┤

│ CENTER  │                 │       │ CENTER  │                 │

│         ├─────────────────┤       │         ├─────────────────┤

│  REAR   │                 │       │  REAR   │                 │

├─────────┼─────────────────┤       ├─────────┼─────────────────┤

│ THRUST  │                 │       │ THRUST  │                 │

└─────────┴─────────────────┘       └─────────┴─────────────────┘

ENGINE - LINER DIAMETER RECORD

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Measure TOP .4 to .6 inches from top.

2.	Measure CENTER 2.6 to 2.8 inches from top.

3.	Measure BOTTOM 4.5 to 4.7 inches from top.

┌─────────┬──────────────────────────┬──────────────────────────┐

│         │      LONGITUDINALLY      │      TRANSVERSELY        │

│ CYLINDER│  TOP     CENTER   BOTTOM │  TOP     CENTER   BOTTOM │

├─────────┼────────┬────────┬────────┼────────┬────────┬────────┤

│    1    │        │        │        │        │        │        │

│         ├────────┼────────┼────────┼────────┼────────┼────────┤

│    2    │        │        │        │        │        │        │

│         ├────────┼────────┼────────┼────────┼────────┼────────┤

│    3    │        │        │        │        │        │        │

│         ├────────┼────────┼────────┼────────┼────────┼────────┤

│    4    │        │        │        │        │        │        │

│         ├────────┼────────┼────────┼────────┼────────┼────────┤

│    5    │        │        │        │        │        │        │

│         ├────────┼────────┼────────┼────────┼────────┼────────┤

│    6    │        │        │        │        │        │        │

│         ├────────┼────────┼────────┼────────┼────────┼────────┤

│    7    │        │        │        │        │        │        │

│         ├────────┼────────┼────────┼────────┼────────┼────────┤

│    8    │        │        │        │        │        │        │

└─────────┴────────┴────────┴────────┴────────┴────────┴────────┘

Alan E. Grant 9/90

ENGINE - PISTON DIAMETER RECORD

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Measure TOP just below the oil ring.

2.	Measure BOTTOM .55 to .59 from the lowest point.

3.	Use BOTTOM* dimension in clearance worksheet.

┌─────────┬──────────────────────────┬──────────────────────────┐

│         │ LONGITUDINALLY (With Pin)│ TRANSVERSELY (Across Pin)│

│  PISTON │     TOP         BOTTOM   │     TOP         BOTTOM*  │

├─────────┼─────────────┬────────────┼─────────────┬────────────┤

│    1    │             │            │             │            │

│         ├─────────────┼────────────┼─────────────┼────────────┤

│    2    │             │            │             │            │

│         ├─────────────┼────────────┼─────────────┼────────────┤

│    3    │             │            │             │            │

│         ├─────────────┼────────────┼─────────────┼────────────┤

│    4    │             │            │             │            │

│         ├─────────────┼────────────┼─────────────┼────────────┤

│    5    │             │            │             │            │

│         ├─────────────┼────────────┼─────────────┼────────────┤

│    6    │             │            │             │            │

│         ├─────────────┼────────────┼─────────────┼────────────┤

│    7    │             │            │             │            │

│         ├─────────────┼────────────┼─────────────┼────────────┤

│    8    │             │            │             │            │

└─────────┴─────────────┴────────────┴─────────────┴────────────┘

Alan E. Grant 9/90

ENGINE - LINER/PISTON CLEARANCE WORKSHEET

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Use BOTTOM* dimension from PISTON DIAMETER WORKSHEET.

2.	There should be no variation in the bore diameter if the liner is honed.

3.	Piston/liner clearance should be .0020 to .0024.

┌──────────────────┬────────────────────────────────────────────┐

│                  │     BORE           PISTON *       PISTON   │

│    CYLINDER      │   DIAMETER     -   DIAMETER  =  CLEARANCE  │

├──────────────────┼──────────────┬──────────────┬──────────────┤

│       SMALLEST   │              │              │              │

│   1   LARGEST    │              │              │              │

│                  ├──────────────┼──────────────┼──────────────┤

│       SMALLEST   │              │              │              │

│   2   LARGEST    │              │              │              │

│                  ├──────────────┼──────────────┼──────────────┤

│       SMALLEST   │              │              │              │

│   3   LARGEST    │              │              │              │

│                  ├──────────────┼──────────────┼──────────────┤

│       SMALLEST   │              │              │              │

│   4   LARGEST    │              │              │              │

│                  ├──────────────┼──────────────┼──────────────┤

│       SMALLEST   │              │              │              │

│   5   LARGEST    │              │              │              │

│                  ├──────────────┼──────────────┼──────────────┤

│       SMALLEST   │              │              │              │

│   6   LARGEST    │              │              │              │

│                  ├──────────────┼──────────────┼──────────────┤

│       SMALLEST   │              │              │              │

│   7   LARGEST    │              │              │              │

│                  ├──────────────┼──────────────┼──────────────┤

│       SMALLEST   │              │              │              │

│   8   LARGEST    │              │              │              │

└──────────────────┴──────────────┴──────────────┴──────────────┘

Alan E. Grant 9/90

ENGINE - PISTON GROOVE/RING CLEARANCE RECORD

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Clearance must be .0008 to .0020.

┌─────────┬─────────────────┬─────────────────┬─────────────────┐

│  PISTON │    TOP RING     │  CENTER RING    │    OIL RING     │

├─────────┼─────────────────┼─────────────────┼─────────────────┤

│    1    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    2    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    3    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    4    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    5    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    6    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    7    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    8    │                 │                 │                 │

└─────────┴─────────────────┴─────────────────┴─────────────────┘

ENGINE - PISTON RING GAP RECORD

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Piston ring gap, top and center must be .0177 to .0217.

2.	Piston ring gap, oil scraper must be .0138 to .0157.

┌─────────┬─────────────┬─────────────┬────────────┬────────────┐

│  PISTON │  TOP RING   │ CENTER RING │  OIL RING  │  OIL RING  │

├─────────┼─────────────┼─────────────┼────────────┼────────────┤

│    1    │             │             │            │            │

│         ├─────────────┼─────────────┼────────────┼────────────┤

│    2    │             │             │            │            │

│         ├─────────────┼─────────────┼────────────┼────────────┤

│    3    │             │             │            │            │

│         ├─────────────┼─────────────┼────────────┼────────────┤

│    4    │             │             │            │            │

│         ├─────────────┼─────────────┼────────────┼────────────┤

│    5    │             │             │            │            │

│         ├─────────────┼─────────────┼────────────┼────────────┤

│    6    │             │             │            │            │

│         ├─────────────┼─────────────┼────────────┼────────────┤

│    7    │             │             │            │            │

│         ├─────────────┼─────────────┼────────────┼────────────┤

│    8    │             │             │            │            │

└─────────┴─────────────┴─────────────┴────────────┴────────────┘

ENGINE - PISTON/PIN CLEARANCE WORKSHEET

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Clearance must be .??? maximum.

2.	Excessive clearance requires new pistons and pins as oversized pins are not available.

┌─────────┬─────────────────────────────────────────────────────┐

│         │      BORE              PIN                          │

│ CYLINDER│    DIAMETER     -    DIAMETER     =    CLEARANCE    │

├─────────┼─────────────────┬─────────────────┬─────────────────┤

│    1    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    2    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    3    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    4    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    5    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    6    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    7    │                 │                 │                 │

│         ├─────────────────┼─────────────────┼─────────────────┤

│    8    │                 │                 │                 │

└─────────┴─────────────────┴─────────────────┴─────────────────┘

Alan E. Grant 9/90

ENGINE - COMPRESSION/LEAKDOWN RECORD

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

NOTES:

1.	Compression at operating		Good, over		11-12 bar

temperature, throttle		Normal			9-10 bar

open, starter speed			Bad, lower than	8 bar

┌──────────┬─────────────────┬─────────────────┐

│ CYLINDER │   COMPRESSION   │  LEAKDOWN (%)   │

├──────────┼─────────────────┼─────────────────┤

│    1     │                 │                 │

│          ├─────────────────┼─────────────────┤

│    2     │                 │                 │

│          ├─────────────────┼─────────────────┤

│    3     │                 │                 │

│          ├─────────────────┼─────────────────┤

│    4     │                 │                 │

│          ├─────────────────┼─────────────────┤

│    5     │                 │                 │

│          ├─────────────────┼─────────────────┤

│    6     │                 │                 │

│          ├─────────────────┼─────────────────┤

│    7     │                 │                 │

│          ├─────────────────┼─────────────────┤

│    8     │                 │                 │

└──────────┴─────────────────┴─────────────────┘

ENGINE - CAM ADJUSTMENT WORKSHEET

DATE:							  1   2   3   4

┌─────────────┐

CAR:							FRONT │    ENGINE   │ REAR

└─────────────┘

ENGINE:							  8   7   6   5

Maserati V8 engine camshafts are infinitely adjustable.

Intake camshaft  67000

European specs:  Valve open   40 deg B.T.D.C. to 80 deg A.T.D.C.

Emission specs:  Valve open   52 deg B.T.D.C. to 68 deg A.T.D.C.

Exhaust camshaft 67500

European specs:  Valve open   54 deg B.T.D.C. to 22 deg A.T.D.C.

Emission specs:  Valve open   63 deg B.T.D.C. to 13 deg A.T.D.C.

Reference articles:

CAM RING            CAM SPROCKET

TECHNICAL:  The cam spline has 32 teeth.  Each increment is 11.25 degrees.  The cam sprocket has 12 holes, 30 degrees apart.  The cam ring has 11 holes, 32.73 degrees apart.  Calculations show that minimum adjustment is 1.02 degrees in 3 tooth increments.

Refer to the excellent technical article by Stan Bishop in the ________ MOCNA Quarterly that describes setup and adjustment of cams.  This article

Set the intake cam to .051 or exhaust cam to .067 . . .

Locate the dowel pin in any convenient sprocket hole and centerpunch the sprocket adjacent to the hole for repeatability. Slide the cam ring onto the splines and find the hole that comes closest to fitting over the pin.

If the cam ring must rotate more clockwise (CW) to fit over the pin, turn it 3 teeth counterclockwise (CCW) and, if necessary, repeat in 3 teeth increments until it fits.

Tighten the cam nut, retard and advance the engine to bring it back to TDC.  Check the dial indicator for valve travel.  It will be incorrect because of chain and sprocket tensioning.  Restrain the cam, remove the sprocket nut and mark the cam ring pin location with a black marker.

If the cam must advance