HARDWARE & TRIM  1

METRIC FASTENERS

Nomenclature:  M8 x 1.25 x 30  =  Diameter x pitch x length

(In millimeters)

────────── PITCH ─────────

DIA		STANDARD	JAPANESE	 FINE

M 1.6	0.35

M 2		0.4

M 2.5	0.45

M 3		0.5

M 3.5	0.6

M 4		0.7

M 4.5	0.75

M 5		0.8

M 6		1.0

M 6.3	1.0

M 7		1.0

M 8		1.25				1.0

M 9		1.25				1.0

M 10		1.5		1.25		1.0

M 12 	1.75		1.25		1.5

M 14		2.0				1.5

M 16		2.0				1.5

M 18		2.5				1.5

M 20		2.5				1.5

M 24		3.0				2.0

M 30		3.5				2.0

M 36		4.0				2.0

STRENGTH GRADES (Rough equivalent):

PROPERTY CLASS

METRIC		SAE GRADE

4.6			 Grade 1

4.8			 Grade 1

5.8			 Grade 2

8.8			 Grade 5

9.8			 Grade 7

10.9			 Grade 8

12.9			 ?

HARDWARE & TRIM  2

PARTS - GHIBLI

Engine comp. chrome edge			J.C. Whitney #12-5825A

PARTS - BORA

Side rubber trim clip	33/35	Restoration Specialty #10744

Trunk lid gas strut		35/78	Porsche 911, 912 ?  Perf. Prod.

Front:  #911.511.331.20

Rear:   #901.512.331.20

DEALERS

ACS Products					Bob Irwin, Aircraft cables,

1585 Copper Drive				fittings (Emer. brake cables)

Lake Havasu City, AZ 86403

602-855-8613

Aircraft Spruce & Specialty Co.	AN/MS hardware, custom hoses,

201 W. Truslow Avenue			control cables, oil coolers,

Fullerton, CA 92632				tools, rod ends, control cables

714-870-7551

800-824-1930

Frontier Fastener &				Hardware: chrome, stainless,

Industrial Supply				nickel, brass, black oxide

3296 W. Sussex Way				Catalog $2

Fresno, CA 93722

209-228-8602 CA

800-742-8445

Gardner-Westcott Company			Hardware, chrome & stainless

30958A Industrial Road			steel

Livonia, MI 48150

313-427-0760

Hillco Performance Fasteners		USS, SAE, Grade 8, SST, Metric

10515 Stanford

Garden Grove, CA 92640

714-534-6882

McMaster Carr Supply Company		Pop rivets

P.O. Box 54960

Los Angeles, CA 90054-0960

213-692-5911

HARDWARE & TRIM  3

DEALERS (CONTINUED)

Metro Moulded Parts, Inc.		Trim clips

11610 Jay Street

PO Box 33130

Minneapolis, MN 55433

MSC Fasteners					Stainless steel, free catalog

P.O. Box 13334

Pittsburgh, PA 15243

412-341-8989

Newco Products					Metric fasteners, bolts, nuts,

18050 Saticoy					washers, interior & machine

Reseda, CA					screws, black braided hose,

818-344-4110					hose clamps, elec. terminals

Performance Products			Porsche gas struts

16129 Leadwell Street

Van Nuys, CA 91406-3488

800-423-3173 USA

800-272-3906 CA

818-787-7500

Pro Antique Auto Parts			Catalog $2

Dept MS

50 King Spring Road

Windsor Locks, CT 06096

203-623-8274

R & E Fastener Company			440 Page catalog $4

P.O. Box 5120

Sparks, NV 89432

800-443-8111

Restoration Specialties & 		Catalog $2

Supplies, Inc.

P.O. Box 328, Dept. A

Windber, PA 15963

814-467-9842

Rick's Automotive Parts

2754 Roe Lane

Kansas City, KS 66803

800-255-4100

913-722-0488 Tech

HARDWARE & TRIM  4

DEALERS (CONTINUED)

South Coast Specialties Co.		Stainless steel fasteners,

P.O. Box 8173					catalog $2

Fountain Valley, CA 92728

714-850-9303

Tioga Stainless				Hardware, Catalog 80G $.25

6 Deborah Drive

Appalachain, NY 13732

?

Totally Stainless				US & metric fasteners

RD #1, Box 1219				120 pg catalog $2.00

Brodbecks, PA 17329

800-767-4781

J.C. Whitney & Co.

1917-19 Archer Avenue

P.O. Box 8410

Chicago, IL 60680

312-431-6102

HARDWARE & TRIM  5

THREAD REPAIR INSERTS

Helicoil Corporation

Metric Coarse Thread Repair Kits

Thread                 Kit      Inserts     Tap      Installation    Insert      Insert    Drill

Size        Length     P/N      Per Kit     P/N        Tool P/N      Pkg P/N    Quantity   Size

M3x0.5	 4.5	5403-3	12	2087-3	7751-3	R1084-3	12	1/8

M20x2.5	 5.3	5403-3.5	12	2087-3.5	7751-3.5	R1084-3.5	12	#27

M4x0.7	 6.0	5403-4	12	2087-4	3747-4	R1084-4	12	11/64

M5x0.8	 7.5	5403-5	12	2087-5	3747-5	R1084-5	12	13/64

M6x1	 9.0	5403-6	12	2087-6	3747-6	R1084-6	12	1/4

M7x1	10.5	5403-7	12	2087-7	3747-7	R1084-7	12	9/32

M8x1.25	12.0	5403-8	12	2087-8	3747-8	R1084-8	12	21/64

M10x1.5	15.0	5403-10	12	2087-10	3747-10	R1084-10	12	13/32

M12x1.75	18.0	5403-12	12	2087-12	3747-12	R1084-12	 6	31/64

M14x2	21.0	5403-14	12	2087-14	3747-14	R1084-14	 6	37/64

M16x2	24.0	5403-16	 6	2087-16	3747-14	R1084-16	 6	21/32

M18x2.5	27.0	5403-18	 6	2087-18	3747-18	R1084-18	 6	47/64

M20x2.5	30.0	5403-20	 4	2087-20	3747-20	R1084-20	 3	13/16

M12x1.75	18.0	5403-12	12	2087-12	3747-12	R1084-12	 6	31/64

M14x2	21.0	5403-14	12	2087-14	3747-14	R1084-14	 6	37/64

M16x2	24.0	5403-16	 6	2087-16	3747-14	R1084-16	 6	21/32

M18x2.5	27.0	5403-18	 6	2087-18	3747-18	R1084-18	 6	47/64

M20x2.5	30.0	5403-20	 4	2087-20	3747-20	R1084-20	 3	13/16

Metric Fine Thread Repair Kits

Thread                 Kit      Inserts     Tap      Installation    Insert      Insert    Drill

Size        Length     P/N      Per Kit     P/N        Tool P/N      Pkg P/N    Quantity   Size

M8x1	12.0	5404-8	12	4984-8	4835-8	R4255-8	12	21/64

M10x1	15.0	5404-10	12	4984-10	4835-10	R4255-10	12	13/32

M10x1.25	15.0	5405-10	12	4944-10	4640-10.1 R4649-10	12	13/32

M12x1.25	18.0	5405-12	12	4944-12	4640-12	R4649-12	 6	31/64

M12x1.5	18.0	5406-12	12	4976-12	4973-12	R3745-12	 6	31/64

M14x1.5	21.0	5406-14	12	4976-14	4973-14	R3745-14	 6	9/16

M16x1.5	24.0	5406-16	 6	4976-16	4973-16	R3745-16	 6	21/32

M18x1.5	27.0	5406-18	 6	4976-18	4973-18	R3745-18	 6	23/32