STORAGE  1

RUST PREVENTATIVE FOR COMBUSTION CHAMBER

Exxon						Rust Ban 357

Shell						VSI circulating oil

Mobil						Mobilarma 524

Gulf							No-rust engine oil grade 2

Valvoline 					Tectyl 930

DESICCANT

Silica Gel Desiccant Products Co.	Calcium Chloride, Dihydrate,

734 East Hyde Park Blvd.			Granular

P.O. Box 4627

Inglewood, CA 90302

213-673-5196

RODENT CONTROL

Place stalks of fresh mint, tied into bundles, in the interior, engine compartment and trunk.  Rodents don't like the smell.  Also prevents a musty odor.

ARTICLES

MIE		Sept-Oct 1981			Storage

MOCNA	Summer 1982			Storage

CAR*		December 1993			Storage

* Classic Auto Restorer

STORAGE  2

SHUTDOWN PROCEDURE

1.	Fill gas tank full

2.	Drain oil hot

3.	Fill with new oil

4.	Lubricate

5.	Fill each cylinder with 2 tablespoons of oil, install plugs,    turn engine over

6.	Seal air intake

7.	Seal exhaust

8.	Loosen all belts

9.	Wash car

10.	Wax - leave white

11.	Put silica gel or calcium chloride inside

12.	Petroleum jelly or oil on chrome

13.	Remove battery - place on slow charge

14.	Car on blocks with weight off springs and tires

15.	Fill transmission full

DURING STORAGE

1.	Rotate driveshaft 2 turns monthly to lube trans and differential or gears will rust

STARTUP

1.	Drain oil

2.	Fill with new oil

3.	Remove spark plugs

4.	Disconnect fuel pumps

5.	Disconnect transistor ignition

6.	Turn engine over to build oil pressure

7.	Reinstall spark plugs

8.	Reconnect fuel pumps

9.	Open fuel line

10.	Ground coil

11.	Pump fuel into can

12.	Close fuel line

13.	Install coil wire

14.	Reconnect transistor ignition

15.	Start car

16.	Drive 5 miles at medium speed

17.	Watch rubber type parts