TORQUE SPECIFICATIONS  1

GRADES (PROPERTY CLASS)	│   STRENGTH

│

Metric	SAE Grade		│   SAE = PSI (pounds per square inch)

4.6		   1			│   Metric = MPa (Mega Pascal)

4.8		   1	          │

5.8		   2			│   1 MPa  = 145 psi

8.8		   5			│   7 MPa  = 1000 psi

9.8		   7           │

10.9		   8           │

12.9		Special        │

TORQUE VALUES

1.	Torque values are for ferrous metal (steel).  Non-ferrous metals have different values (copper, stainless steel, etc.)

2.	Torque values are shown for turning the nut while holding the head of the bolt.  If the application demands tightening by the bolt head, increase the value shown by 20% (multiply by 1.20).  This will allow for the natural torsional twist of the bolt shank.

3.	Torque values are calculated at 75% of proof load.  This provides a 25% safety factor.

4.	All dry torque values are given for the "as received" condition, plated or not.

5.	Lubricated torque values are calculated based on applying copper anti-seize compound to the threads before assembly.  Adjust the torque value for other lubricants.

TORQUE FORMULA

KDW

T = --- = FOOT-POUNDS

12

T = Torque

K = Friction factor

0.30 for nonplated fasteners, black finish

0.20 for electrodeposited zinc coatings

0.15 - 0.18 for oils

0.16 for cadmium electrodeposited coatings

0.12 for copper anti-seize compound

D = Nominal bolt diameter in decimal inches

W = Clamp load in pounds (75% x proof load)

TORQUE SPECIFICATIONS  2

METRIC HEX HEAD CAP SCREWS (REGULAR PITCH)

CLASS  8.8	CLASS  9.8	CLASS 10.9	CLASS 12.9

DRY  LUBED	DRY  LUBED	DRY  LUBED	DRY  LUBED

Values in inch-pounds (Divide by 12 for foot-pounds)

M4	27.5	17	30	18	38.5	24	53	32.5

M5	56.5	33.5	61	37	78	47	107	65

M6	95	57.5	103	61	132	79	180	109

Values in foot-pounds (Multiply by 12 for inch-pounds)

M8	19	12	21	13	27	16	37	22

M10	39	23	42	25	53	32	73	44

M12	67	40	73	44	92	55	127	76

M14	107	64	116	69	148	89	203	122

M16	167	100	181	108	230	138	316	190

M20	325	195	352	211	449	269	617	370

M24	562	337	609	366	775	465	1066	640

M30	1117	670	1210	726	1540	924	2188	1271

TORQUE SPECIFICATIONS  3

NOTE:  Most data is from QUATTROPORTE III manual

DESCRIPTION					THREAD SIZE   KGM   FT/LB

(x12=In/Lb)

ENGINE

Head bolts					M12 x 1.5	   11.04	   80

Front head nut					M 8 x 1.25	  2     14

Cam cover nut					M 6 x 1   	1.5   10.5

Camshaft sprocket nut			M22 x 1 		 22    159

Main bearing nut				M 8 x 1.25       3     22

Main bearing nut				M12 x 1      10.35     75

Connecting rod nut (Ghibli)		M		    8.04     58

Connecting rod nut (Bora)		M			  7	   51

Camshaft support nut			M 8 x 1.25	  3     22

Spark plug					M14 x 	   2.5-3  20-22

Timing chain tensioner nut		M12 x 1.25     .15      1

Timing chain rubbing block		M		     .35	 2.53

Oil sump nut					M 7 x 1	     1.5     11

Crank pulley bolt (Ghibli)		M12 x 1.25   11.04	   80

ELECTRICAL

Alternator pulley				M14 x 1.5	 3.5-4.5	26-33

Starter mount nut				M11 x 1.5		  8     58

FUEL SYSTEM

Carburetor mount nut			M 8 x 1.25   0.8-1    6-7

Pump mount					M 6 x 1		2-3	15-22

EXHAUST SYSTEM

Manifold to cylinder head nut		M 8 x 1.25	2.5	   18

CLUTCH AND FLYWHEEL

Flywheel/crankshaft nut (Ghibli)	M 9 x 1.25       3     22

Flywheel/crankshaft bolt (Bora)	M10 x 1.25	 16	  120

Clutch pressure plate bolt		M10 x 1.25   2.9-3  21-22

TORQUE SPECIFICATIONS  4

DESCRIPTION					THREAD SIZE   KGM   FT/LB

(x12=In/Lb)

TRANSMISSION, ZF

Transmission to mount			M11 x 1.5		  7	   51

Mount to frame					M12 x 1.75	  6     44

Clutch housing to engine nut		M 8 x 1.25   3-3.2  22-23

Clutch housing to transmission	M10 x 1.5       11     80

DRIVESHAFT

Driveshaft to transmission		M11 x 1.5	      11     80

Driveshaft to differential		M11 x 1.5		  6     44

FRONT AXLE

A-frame						M12 x 1.25      12     87

Inner support of ball joint		M			  8     58

Stabilizer bar to frame			M		   2.8-3  20-22

Stabilizer bar to support		M		   2.8-3  20-22

Shock absorber top				M12 x 1.25      12     87

Shock absorber bottom			M12 x 1.25      10     72

Wheel bearing nut				M16 x 1.5	     .45    3.3

Ball joint to stud axle			M		      11     80

Link rod to bar and to suspension	M12 x 1.25	  8     58

STEERING & WHEEL GEOMETRY

Steering gearbox to frame		M10 x 1.5		  8     58

Steering lever rod adjustment nut	M			  7     51

Pitman arm castle nut			3/4-16		7-8  51-58

Steering shaft universal joint	M 7 x 1		  3     22

Steering column to frame			M 8 x 1.25	  3     22

Steering wheel nut				M16 x 1	   3.5-4  25-29

Pump screws					M			3.5	   25

Pump pulley nut				M			  9	   66

REAR AXLE

Shock absorber					M16 x 1.5		7.6     55

Rim fixing hub to axle shaft		M		      16    116

TORQUE SPECIFICATIONS  5

DESCRIPTION					THREAD SIZE   KGM   FT/LB

(x12=In/Lb)

BRAKE CIRCUIT

Caliper mounting screw			M12 x 1.75	  8     58

Caliper mounting screw			M10 x 1.25	  8	   58

Housing to front wheel hub screw	M12 x 1.25	  8     58

Disc to housing screw			M 5 x 0.8	    0.85      6

WHEEL

Wheel bolts					M		     8-9  58-65

AIR CONDITIONING & HEATING SYSTEM

Compressor screw				3/8-16	   3-3.5  22-25

Condenser screw				M		   1.5-2  11-15