## Bora Technical Details

Engine

Cylinders		90 deg V8

Bore			94 mm (3.70 in.).		Displacement

Stroke			85 mm (3.35 in.).		4719 cc (288 cu. in.).	 Original     1971-76.

89 mm (3.50 in.). 	4930 cc (300.8 cu. in.) 	All Boras   1977-79.

US version 1973-79.

Valve gear		OHV with four overhead camshafts (DOHC)

Compression ratio	8.5:1 	Original Version 1971-1972

US Version 1973-79

8.75:1	All Boras outside US 1973-79

Carburetors		4 Weber 42DCNF

Maximum Power	Horsepower		Torque

310 hp @ 6000 rpm. 4.7 	325 ft lb. @ 4200 rpm	original version 1971-76

280 hp @ 5800 rpm. 4.9 	3l0 ft lb. @ 3500 rpm	US version 1973-79

320 hp @ 5500 rpm  4.9 	335 ft lb. @ 4000 rpm	outside US 1977-79

Clutch:			Diaphragm-type, single dry plate, with hydraulic actuation.

Gearbox:	ZF  transaxel with five forward speeds and reverse, integral with final drive unit Synchromesh on all forward speeds. Hypoid bevel crown wheel and pinion. Limited-slip differential 1/3 or 2/3 optional. Same unit as utilized on GT40 small block, Pantera, Monteverdi Hai, Iso Rivolta Varedo, BMW M1 Ratios:

5th	0.74:1

4th	0.85:1

3rd	1.04:1

2nd	1.52:1

1st	2.58:1

Rev.	2.86:1

Final Drive Ratio	3.77:1   Standard on non US cars.

4.22:1   Standard on US cars  3.77 optional.

Body and Chassis construction:  	All-steel monocoque body shell, mid engine, isolation mounted/anchored rear sub-frame, rear wheel drive.

Suspension:	Front/Rear Independent, wishbones and coil springs; telescopic shock absorbers; anti-roll bars.

Steering:	Rack and pinion, 3 turns lock to lock with a ratio of 17.7:1

Brakes :		High pressure Hydraulic Citroen utilizing LHM mineral fluid, a high pressure pump/regulator and 3 accumulators.

Front : 11.8” Ventilated rotors and four (4) piston calipers.

Rear:   11.0” Ventilated rotors and  three (3) piston calipers.

Mechanical linkage for hand brake on rear rotors.

Wheels:		Campagnolo cast magnesium light alloy. 71/2  J15 (front and rear).

Tires:		Michelin XWX 205/70 VR15 front  non US cars.

Michelin XWX 215/70 VR 15 front  US cars and rear all cars.

Special Equipment:	Air-conditioning, Power windows, Stereo cassette FM tuner with 2 speakers and concealed power antenna, Hydraulic power tilting divers seat, Hydraulic operated gliding foot pedals and concealed headlamps, 12 V 55 Amp-hour alternator, Quartz-iodine headlamps.

Fuel tank capacity:		20 Gal with 2 fuel pumps (1 as backup)

Dimensions:

Wheelbase			102.4 in. (2600 mm).

Track front			58.0 in. (1475 mm).

Track rear			56.9 in. (1447 mm).

Overall length			170.7 in. (4335 mm).

Overall width			69.6 in. (1768 mm).

Overall height			44.8 in. (1138 mm).

Ground clearance			4.5 in. (114 mm).

Turning circle			34.4 ft (10.5 m).

Dry weight			3350 lb. (1520 kg) Original, 1971-79.

3405 lb. (1545 kg) US version 1973-74

3484 lb. (1580 kg) US version 1975-79.

Performance	Top gear mph per 	28.9.		1971-79  3.77 gears						     1000 rpm		25		1973-79  4.22 gears

Maximum speed	174 mph (theoretical)	4.7 litre 1971-76   3.77 gears @ 6000rpm

4.9 litre 1977-79   3.77 gears

168 mph 		Factory claimed top speed.

150 mph US version 	4.9 litre 1973-79   4.22 gears @ 6000rpm

0-60 mph accel.	6.5			4.7 litre 1971-76  3.77 gears

6.2			4.9 litre 1977-79  3.77 gears

6.2 US			4.9 litre 1973-79  4.22 gears w full smog

5.7			4.9 litre                4.22 gears

w/o smog  and with European cam timings

Quarter Mile	14.5 @ 98 mph		4.7 litre 1971-76 3.77 gears

14.3    			4.9 litre 1977-79 3.77 gears

14.9 US			4.9 litre 1973-79 4.22 gears

w full smog tune

14.1 @ 102 mph US	4.9 litre 1973-79 4.22 gears

w/o smog and  with European cam timings

1974 Maserati-Thepenier Bora Group 4 modifications: serial # 3001

Price:	1 million (lire (1974)

Structure steel unibody with steel and fiberglass outer

Rear hatch – aluminum Plexiglas with extensive ventilation holes

Extensive deletion of standard equipment to pare weight

Engine

418  (bhp) @ 5500 rpm

Intake system; 4x2bbl downdraft carburetors (Weber) 50 IDA

Dry sump lubrication

Forced cool air induction

Heads and intake manifold extensively excavated

Larger exhaust system

Copper head gaskets

Increased compression ratio

Dimensions

Curb weight (lbs.): 2550  That’s 800 pounds lighter!

Overall length (in.): 1790

Suspension

Hubs and spindles upgraded

Shocks, springs and sway bars re-calibrated

Wheels: F/R: 15x11 / 15x13 (Campagnolo)

Brakes:  thicker discs with 4 piston Girling F1 racing calipers

*~/~ 3001